# Dawson College

# Electrical Engineering Technology Department

# Introduction to Internet of Things

# Lab Procedure #13: Application Design

## Introduction

## In this lab students will complete a performance-oriented laboratory exercise centred on an example project, Smart Door Parcel Monitor. The first part presents a concise, single-page procedure that revisits techniques previously covered in the course; it is intended to guide you systematically through circuit, code upload, and functional testing. The second part contains a ten-question quiz that will assess your comprehension of the project’s hardware-software interaction and the underlying IoT concepts. Your quiz responses will be evaluated during the lab period and will constitute the formal mark for this activity.

## Addressed competencies

- 02GW Produce technical documentation.

- 02GY Program a connected system or equipment.

## Procedure

Use this one‑page walkthrough to understand and reproduce the project.

- Clone the repository and open the sketch:

  - https://github.com/cmendes-dc/PBL-Intro2IoT

- Install the NewPing library

  - Arduino IDE → Sketch → Include Library → Manage Libraries.

- Build the circuit (see schematic PNG on GitHub):

  - HC‑SR04: Trig→D11, Echo→D10, Vcc→5 V, GND→GND.

  - Active buzzer: +→D5, –→GND.

  - LED: Anode→330 Ω→D6, Cathode→GND.

  - Push‑button: one leg→D4, other→GND (INPUT_PULLUP).

- Upload the sketch to your Arduino and open Serial Monitor to view live distance data.

- Trigger the alert: raise an object until distance > 10 cm; LED and buzzer should activate.

- Press the push‑button; verify the alert mutes (`Alert silenced by button.` shows in Serial).

- Lower the object (distance ≤ 10 cm). LED/Buzzer stay off and system auto‑re‑arms.

- Experiment: change the `threshold` value, observe behaviour.

## Smart Door Parcel Monitor – Laboratory Performance

Make sure you understand the code from the example project very well before proceeding with the questions below. Each question = 1 pts.

- What primary problem does the Smart Door Parcel Monitor aim to solve?

- Which sensor is used to detect the food level in the bowl?

- Name one output device that notifies the owner when the bowl is empty.

- Why is the push-button wired to an INPUT_PULLUP pin instead of using an external resistor?

- Which Arduino library is included to simplify ultrasonic distance measurement?

- What numeric value is assigned to threshold, and what does it represent?

- State the two conditions checked before the code decides the bowl is empty.

- What variable stores the mute/override state after the push-button is pressed?

- Explain briefly what happens in the code when the bowl is refilled (distance ≤ threshold).

- Why is a 330 Ω resistor connected in series with the LED?